MAKİNA MÜHENDİSLİĞİ BÖLÜMÜ ME 492 ENGINEERING PROJECT

PROJECT ANNOUNCEMENT

Spring 2026

Title: Experimental Determination of Vibrational Properties of A rectangular plate WITH PROPERTIES VARYING ALONG ONE EDGE

Ref Code: Akaltan-F26-2

Type: Group

Status: New

Availability: 2

Project Description:

Experimental: A rectangular thin plate with properties varying along one edge (i.e. a metal plate with varying thickness or a wooden plate etc.) will be placed under an appropriate speaker. BCs will be modeled in a way that the two opposite edges along the non-varying axis being simply-supported and the remaining boundary conditions can be any combination of classical boundary conditions (clamped, simply-supported, free). A granular material will be poured onto the plate. The plate will be exposed to pure sine sound waves (forced vibration) with an increasing frequency, hence a frequency sweep. Thus, from the formation of Chladni patterns, several natural frequencies of the plate will be determined. There might be additional experiments with other sensors in place.

Finite Element: The plate used in the experimental study will be modeled in ANSYS, FEAP, and another finite element program might be included. Modal analysis will be carried out with the precise data collected and documented by the students. The experimental and finite element results will be compared.

Work Description:

- Design and build the boundary condition fixture and experimental setup.

- Plan and conduct frequency-sweep tests; record and process Chladni patterns.

- Develop FE models (material/geometric properties, proper BCs).

Required Qualifications and Skills:

- Carefulness in experimental work

- Basic knowledge of vibration analysis and finite element analysis

- Some coding might be required (Matlab, Octave)

- The project will be on campus.

Related UN Sustainable Development Goals:

- No.4 Quality Education

Difficulty:		high ( )			medium ( )		low (X)

Time commitment: 	high (X)		medium ( )		low ( )

Estimated expense:	1000 TL