PROJECT ANNOUNCEMENT

Fall 2026

Title: Effect of carbon fiber reinforcement on mechanical and physical properties of Polyamide

Position Ref: HTV

Type: Group/Individual

Status: New

Seats: 1/2 student(s)

Project Description:

Thermoplastic materials are widely used to manufacture of light weight drone,automobile, electrical appliance and white good part. Short carbon fiber reinforced polyamide (SCF/PA) is an important engineering thermoplastic composite that combines the processability of polyamide with the high stiffness and strength of carbon fibers. In this project, students will investigate the relationship between material composition, processing, structure and mechanical performance of SCF/PA composites.

The project aims to understand the characteristics of polyamide as a polymer matrix and carbon fiber as a reinforcement, and to investigate how carbon fiber content, compounding and injection molding conditions influence fiber distribution, orientation and the resulting mechanical properties of the composite.

Experimental Work

The experimental part of the project will consist of three main steps:

1. Compounding

Different amounts of short carbon fiber will be incorporated into a selected polyamide resin using a twin-screw compounding extruder. Several carbon fiber concentrations will be prepared in order to investigate the effect of reinforcement level on the final properties.

2. Sample Preparation

The compounded SCF/PA materials will be processed using an injection molding machine to produce standard test specimens. The influence of processing conditions and fiber orientation on the resulting specimens should be considered.

3. Testing and Characterization

The injection-molded specimens will be tested to determine their mechanical properties. Depending on available laboratory facilities, testing may include; Tensile strength and tensile modulus, Elongation at break, Flexural strength/modulus, Impact strength, Hardness, Density

The experimental results should be analyzed as a function of carbon fiber content and compared with the unreinforced PA reference material.

Required Qualifications and Skills:

•	Delicate hand skill to production machinery usage

•	Carefulness in experimental work

•	Basic knowledge of materials

Related UN Sustainable Development Goals:

•	No.9 Build resilient infrastructure, promote inclusive and sustainable industrialization and foster innovation

Difficulty:		high (-)	medium (X )		low ( )

Time commitment: 	high (-)	medium ( X)		low ( )